PROJECT TITLE:

CAN-Controlled Automotive Information Display System

AIM:

The aim of this project is to design and implement an embedded system that monitors and displays important vehicle parameters such as:

- Engine Temperature

- Fuel Level

- Indicator Status

Communication between nodes is achieved using the Controller Area Network (CAN) protocol, making the system reliable and suitable for automotive applications.

------------------------------------------------------------

PROJECT INSIGHT:

This project provides hands-on experience in:

- Embedded C Programming

- LPC2129 Microcontroller Architecture

- ADC (Analog to Digital Converter)

- CAN Communication Protocol

- GPIO, External Interrupts, and RTC

------------------------------------------------------------

HARDWARE REQUIREMENTS:

1. LPC2129 Microcontroller

2. CAN Transceiver (MCP2551)

3. LCD Display

4. LEDs

5. Switches

6. DS18B20 Temperature Sensor

7. Fuel Gauge Sensor

8. USB to UART Converter

------------------------------------------------------------

SOFTWARE REQUIREMENTS:

1. Embedded C Programming

2. Keil C Compiler

3. Flash Magic Software

------------------------------------------------------------

SYSTEM NODES USED:

1. MAIN NODE

2. INDICATOR NODE

3. FUEL NODE

------------------------------------------------------------

WORKING PRINCIPLE:

MAIN NODE:

- Continuously reads:

- Engine Temperature

- Real-Time Clock (Date, Time, Day)

- Displays all data on the LCD.

- Receives fuel data from the Fuel Node through CAN.

- Sends indicator control signals to the Indicator Node using CAN based on external interrupts.

INDICATOR NODE:

- Continuously waits for CAN data from the Main Node.

- Based on received data, it controls the indicator LEDs.

FUEL NODE:

- Continuously reads the fuel sensor value using ADC.

- Sends fuel percentage data to the Main Node via CAN.

------------------------------------------------------------

IMPLEMENTATION STEPS:

1. Create a project folder.

2. Test LCD for:

- Character display

- String display

- Integer display

3. Test on-chip ADC using a potentiometer.

4. Develop fuel gauge reading logic using ADC.

5. Develop temperature reading logic using DS18B20.

6. Test external interrupts EINT0 & EINT1.

7. Test RTC functionality.

8. Test CAN communication between boards.

9. Integrate all modules into the final application.

------------------------------------------------------------

OUTPUT:

The system successfully displays:

- Engine Temperature

- Fuel Percentage

- RTC (Time, Date, Day)

- Indicator ON/OFF Status

------------------------------------------------------------

APPLICATIONS:

- Automotive Dashboard Systems

- Vehicle Monitoring Systems

- Embedded Networking Applications

- CAN Protocol Training Projects

------------------------------------------------------------

AUTHOR:

Developed by: Meghana Konepalli

------------------------------------------------------------

STATUS:

Project Successfully Implemented and Tested